CIPC Residues Monitoring Group

for the UK potato industry

Chair: Adrian Cunnington   Secretary: Graham Bannister

Guidelines for further reducing risk of chlorpropham (CIPC) residue contamination

Objective:	To raise awareness of on-going risk and further reduce CIPC pick-up from potato store fabrics, structures and equipment to minimise residues found on future stored crops

As part of the CRMG’s surveillance activity undertaken to support the current temporary Maximum Residue Level (tMRL) for chlorpropham (CIPC), the Group is urging all growers and store managers with stores that have any history of CIPC treatment to focus on cleaning between storage seasons as this is key to reducing on-going contamination risk. Our work has shown that around one in seven potato stores in the UK still carries a risk of CIPC residues remaining from treatments made prior to CIPC withdrawal in 2020.

The steps outlined below can help to reduce traces of the sprout suppressant in previously-treated potato stores. Whilst there will continue to be a temporary Maximum Residue Level in potatoes (currently 0.35 mg/kg), other non-target crops have a lower tolerance with many set at the limit of detection (typically 0.01 mg/kg).

There is also a possibility that changes in legislation linked to the UK-EU Sanitary and Phytosanitary (SPS) alignment agreement, expected to take effect in mid-2027, may result in changes to the UK tMRL. The European tMRL for chlorpropham is currently 0.20 mg/kg.

CLEANING GUIDANCE

  - Although stores will not have been treated in recent seasons, it is possible for residual CIPC, particularly in inaccessible parts of the store, to still pose a risk to the saleability of your crop. Our data show that stored crops are still picking up measurable levels of CIPC even though it hasn’t been used since 2019/20.

  - HSE advises that all stores with a history of CIPC use should continue to be cleaned annually to reduce the levels of residues still being found in previously-treated stores.

  - Cleaning activities must include appropriate safety measures: always use personal protection equipment (PPE) and pay attention to safety when working at height. Keep written records and pictures of what has been cleaned, how and when. Buyers of the potatoes may request a record of cleaning activities, especially if an exceedance of the tMRL is recorded.

  - Dry cleaning methods are preferred; always start with them. Only use water if no suitable dry cleaning method is available or if it is not sufficient for complete cleaning. When using complementary wet cleaning, an acceptable method of collecting water and disposing of it appropriately must be used to avoid dispersing contaminated waste water into the environment.

  - Care should be taken to minimise re-distribution of any residual CIPC. Any remaining loose materials should be removed promptly, by vacuuming. Sweeping and brushing alone tend to generate dust, which risks re-distribution.

  - Cleaning should be carried out from top to bottom (i.e. roof to floor).

  - CIPC is only slightly volatile. Volatilisation will contribute to removal of CIPC, but this process is very slow. When the store is not in use for potato storage, doors and hatches should be left open to allow for continuous airing.

  - After cleaning, underfloor ducts should be subject to constant movement of air, either by running fans or by natural draft. A low air speed is sufficient.

  - Where possible, remove loose residual waste material by dry cleaning and then move store hardware (eg boxes or above ground ducts etc.) outside so weather action (sun, rain, wind, temperature) can also contribute to CIPC removal.

  - Cleaning efforts should continue to focus on positions within the store of highest contamination risk, like plenums and fans or around historic application points.

CHECK OUT THE INFOGRAPHIC AT THE END OF THIS GUIDANCE

SPECIFICS

Clean all loose materials by brushing + vacuuming together (not in sequence, but always combined) from any critical areas in the store. If stubborn deposits are still apparent, bearing in mind it is over 5 years since CIPC was banned, more aggressive methods (eg wire brushing) may be required.

Wooden boxes

  - Clean any stubborn contaminants by brushing.

  - If box contamination is a concern, try to store boxes outdoors for the entire period that they are not in use. Boxes should ideally not be close-stacked but do always take care to ensure that they are stacked safely.

Hotspots

Exceedance risks from ‘hotspots’ depend on the type and location of historic CIPC application and on the fog distribution within the store when hot-fogging practices were used. Carefully consider the places where CIPC fog was applied or where accumulated deposits may still be found which were not removed by regular cleaning in recent seasons (floor and wall near fogging port, plenum, fans, refrigeration unit, roof-heater fans, anti-condensation fans etc.)

FURTHER INFORMATION

Visit the GB Potatoes website to access cleaning guidance and more information on the work of the CIPC Residues Monitoring Group at:

https://www.gb-potatoes.co.uk/resources